17.02.2016

Exercise instructions:

Create an entry on Benchling using the protocol below (Colony PCR protocol):

1.  Open Benchling (https://benchling.com) and log in.

2. Within your own workspace click the big “+” (Create Project) right next to Projects in your Benchling workspace. Name it as you wish (e.g., “ELN training”)

3. Navigate to your project folder (suitcase icon on the left) and add a new blank Entry (“+”) and name it “Colony PCR protocol”

4. Navigate to the tab “NOTES” on the top to add the information from “Colony PCR protocol” to your entry.

Suggestions:

- Can you make the reaction set up table more interoperable? (e.g., make it possible to calculate the total volume using the SUM function?)

- can you make a new data table for the thermocycling conditions?

Extra!

Create a primer sequence to add to your table

- Under your project, go to “+” and choose “Oligo” and “New DNA/RNA Oligo”

- Give it a name (e.g.,“Open_primer1”)

- Select your project folder

- Choose DNA for nucleotide type and under Schema choose “primer”

- Copy and paste the bases “ATGGCCBCAAGTTGACC”

- To add the link to your new primer to your protocol, navigate back to “Colony PCR protocol”, choose the location (e.g., your data table), and search for your object (in the case the primer) by typing and @ and Open as “@Open” and wait until it finds the right object and click on it.

------

Colony PCR Protocol

- Put all the necessary material to thaw on ice (except DNA polymerase, which remains in the freezer until the very last moment)

- Label PCR tubes and put them on ice

  - Prevents enzymatic activity before the beginning of the PCR and prevents potential DNase to digest the DNA

- Resuspend every colony in 15ul H2O

  - Pick the colony with a tip and put it the water

  - Resuspend the cells by pipetting up and down

  - Makes sure that you keep track of your colony

- Prepare the PCR Mix (number of colonies + 1X/10 colonies)

  - Saves time rather than to do it tube by tube

  - Avoids tube-to-tube variability

  - Avoids pipetting very small volumes (increases error risk)

  - The additional 1X compensates the loss of Mix solution by pipetting

Reaction set up:

| Component | 25 μl reaction | Mix (10X) | Mix (15X) | Mix (20X) | Mix (50X) | Final Concentration |
| --- | --- | --- | --- | --- | --- | --- |
| Nuclease-free water | 14.3 ul | 143 ul | 214 ul | 286 ul | 715 ul | - |
| 5X Standard Taq Reaction Buffer | 5 ul | 50 ul | 75 ul | 100 ul | 250 ul | 1X |
| 10 mM dNTPs | 0.5 ul | 5 ul | 7.5 ul | 10 ul | 25 ul | 200 µM |
| 10 µM Primer 1 | 0.5 ul | 5 ul | 7.5 ul | 10 ul | 25 ul | 0.2 µM (0.05–1 µM) |
| 10 µM Primer 2 | 0.5 ul | 5 ul | 7.5 ul | 10 ul | 25 ul | 0.2 µM (0.05–1 µM) |
| Resuspended colony | 1 ul | - | - | - | - | <1,000 ng |
| MgCl2 (25mM) | 4 ul | 40 ul | 60 ul | 80 ul | 200 ul | 4 mM |
| Flexi Polymerase (5U/ul) | 0.15 ul | 1.5 ul | 2.25 ul | 3 ul | 7.5 ul | 0.75 unit |
| Total Volume | 25 ul | 250 ul | 375 ul | 500 ul | 1250 ul |  |

- Note that if you just want to run the PCR product on a gel, you can use the green Flexi 6X buffer, which already contains a dye for electrophoretic migration

- Add the DNA polymerase to the Mix at the end

  - Make sure to keep it on the blue cold rack (which remains in the freezer)

- Put 24ul of PCR Mix in every PCR tube

- Add 1ul of suspended cells in every tube

- Start the PCR

Thermocycling conditions:

- Initial denaturation for 10 minutes at 95ºC;

- 30 cycles of amplification :

  - denaturation at 95ºC for 20 seconds

  - primer annealing : 55ºC for 20 seconds, (temperature depends on the primers Tm)

  - extension at 72ºC for 1min/kb;

- Final extension at 72ºC for 5 minutes.

- Hold at 4ºC

Enjoy your PCR!